# September 14-16, 2023, Institute of Chemical Technology, Mumbai

Extraction of phenolics from Spilanthes Acmella: Kinetic studies using Peleg’s model

Nitin T. Satao, Komal V. Mahindrakar, Virendra K. Rathod1

1Department of Chemical Engineering, Institute of Chemical Technology, Matunga (E), Mumbai-400019.

E-mail address: vk.rathod@ictmumbai.edu.in

## Introduction:

Spilanthes Acmella (SA), also known as "Akkarkara" or "toothache plant," belonging to the Asteraceae family, is a valuable medicinal herb found in tropical regions. The phenolics present in the plant have widespread applications in the pharmaceutical, food, and personal care industries. However, the low concentration of phenolics in plant material requires the development of a suitable extraction method utilizing chemical engineering tools. This study aims to standardize an extraction technique and investigate the impact of various extraction parameters on the yield of phenolic extracts. By employing batch extraction and Peleg's model to explore extraction kinetics, we aim to obtain consistent, high-quality phenolic extracts.[1]

## Material and Methods:

2.1. Materials

The dried flowers of Spilanthes Acmella were procured, and residual moisture was removed in the oven and ground to powdered form. AR-grade extracting solvents, standard Gallic acid, sodium carbonate, and Folin - Ciocalteu’s phenol reagent were purchased and deionized water was taken from the DI water plant, ICT, Mumbai.

2.2. Experimental work

2.2.1 Soxhlet extraction

In Soxhlet extraction, a 1 gm sample of SA powder was taken in a thimble and extracted continuously with water as a solvent for 5 hours. Extracts were analyzed and TPC obtained was expressed in mg/g.

2.2.2 Stirred batch extraction

Batch extraction was performed in a 100 ml glass reactor equipped with a pitched blade glass turbine impeller and the extract was analyzed using Folin Ciocalteu’s method by UV-visible spectrophotometer, determining the TPC in SA powder extract in mg/g. The study was conducted on various parameters that affect the efficiency of extraction like the type of solvent, time of extraction, solid-to-solvent ratio, pH value, and temperature. After studying each parameter, we obtained optimized conditions for the final experiment. The experiment was then carried out under these optimized conditions.[2]

2.2.3 Kinetic studies for batch extraction of phenolics

Kinetic modeling was done with Peleg's model to determine TPC extraction kinetics from Spilanthes Acmella using batch extraction. The study aimed to find equilibrium concentration, initial extraction rate, and rate in the later part of extraction.

2.2.4 Total phenolic content determination assay

To determine the total phenolic content of the extract obtained from Soxhlet and batch, the Folin Ciocalteu's method was used. The sample was incubated in the dark at room temperature for 2 hours. The absorbance was measured at 750 nm using Jasco V-630 UV-visible spectrophotometer, after the incubation period. The total phenolic content was determined in mg of GAE/g of the sample.

## Significant Results and Discussion

3.1 Optimization of process parameters and validation of the model

In the present work, the influence of different solvents (methanol, water, ethanol, acetone, and n-butanol), extraction temperatures (30, 40, 50, and 60 °C), solid to solvent ratio (1:10, 1:20, 1:30, 1:40 and 1:50), speed of agitation (100, 200, 300, 400 and 500 rpm), and pH value (4, 5, 6, 7 and 8) on the amount of TPC extracted per g of Spilanthes Acmella powder have been investigated. The experimental values have also been compared with the values obtained from Peleg’s model. To decide the optimum time of extraction, the experiments done with different solvents have been considered and the obtained trends for the variation in the amount of TPC extracted per g of Spilanthes Acmella with time have been given in Fig. 1. As seen in Fig. 1, the extraction of TPC was very fast during the first 15 min as the TPC concentration gradient between the solvent and the plant material is maximum and also due to the ease of extraction from the outer part of the matrix during the initial period. Afterward, the yield of TPC increased gradually with time till 25 min because of the lower concentration gradient and because the remaining TPC is located in the interior part of the plant cell. Based on the observations, it has been established that 30 min is required for giving maximum release of TPC in the case of stirred batch extraction and hence all the experiments related to the understanding of the effect of operating parameters were performed for 1 hour.[3]

3.2 Comparison of different extraction techniques based on yield and economy

The study compared the results of Soxhlet and stirred batch extraction techniques based on the yield of TPC (Table 1) and economy in terms of energy consumption and cost estimation (Table 2). The soxhlet technique uses a reflux mechanism at the boiling point of water, which may result in the degradation of thermolabile compounds during extraction. However, when water was used as a solvent, the Soxhlet technique produced a lower extraction yield compared to stirred batch extraction. Stirred batch extraction is a quicker process that requires less solvent and temperature while producing higher TPC than the Soxhlet technique. In addition, stirred batch extraction was found to be more cost-effective and required less energy than Soxhlet extraction, with costs and energy requirements around 500 times lower than that of the Soxhlet technique (Table 2). Therefore, it has been demonstrated that batch extraction is an economical and efficient method for extracting phenolic compounds from SA.

3.3 Figures and tables

## Table 1: Yields of TPC by different extraction techniques and extraction parameters.

| Extraction techniques | TPC (mg GAE/g) | Time | Water (mL) | Temperature (℃) | Stirring (rpm) |
| --- | --- | --- | --- | --- | --- |
| Soxhlet | 11.59 | 5 h | 180 mL | 100 | NA |
| Stirred batch extraction | 18.91 | 30 min. | 30 mL | 50 | 400 |

Table 2: Energy and cost estimation for extraction of TPC by different extraction techniques.

|  | Energy (104 kJ/kg) | Cost of process (Rs/kg) |
| --- | --- | --- |
| Soxhlet | 454.86 | 110935 |
| Stirred batch extraction | 8.85 | 215.85 |

Figure 1: Effect of solvents on the kinetics of batch extraction of TPC from SA powder at solid to solvent ratio 1:30; extracting temperature, 50C; the speed of agitation of 400 rpm and extracting time, 30 min and Peleg’s model fitting.

## Conclusions:

The work performed to check out whether the extraction of TPC from Spilanthes Acmella is feasible by batch extraction technique and results obtained in terms of the extraction yield of TPC, has shown promising results. The process parameters which affect the ultrasound-assisted extraction yield like extraction time, solid-to-solvent ratio, extraction temperature, speed of agitation, and pH have been optimized. The higher TPC yield of 18.52 mg/g (solvent used water, solid/solvent ratio of 1:30, temperature of 50◦C, agitation speed 400 rpm, and pH of 5) is obtained in just 15 min indicating that the batch technique can be used for extraction TPC from Spilanthes Acmella powder. The kinetic model (Peleg’s model) has been used for the prediction of the amount of TPC in the extract at a given time for all experimental conditions. The model is validated by plotting experimental and predicted values of the amount of TPC in an extract with good agreement.

References

[1]	V. Pandey, V. Agrawal, Efficient micropropagation protocol of spilanthes acmella L. possessing strong antimalarial activity, Vitr. Cell. Dev. Biol. - Plant. 45 (2009) 491–499.

[2]	V.M. Kulkarni, V.K. Rathod, Extraction of mangiferin from Mangifera indica leaves using three phase partitioning coupled with ultrasound, Ind. Crops Prod. 52 (2014) 292–297.

[3]	M.D. Vetal, V.G. Lade, V.K. Rathod, Extraction of ursolic acid from Ocimum sanctum by ultrasound: Process intensification and kinetic studies, Chem. Eng. Process. Process Intensif. (2013).